.help wiener Jul91 stsdas.analysis.restore

.ce

FOURIER NONITERATIVE IMAGE RESTORATION

.sh

1. INTRODUCTION

Fourier noniterative image restoration techniques have as their main

advantage the short computation time they need to produce a solution,

when compared with more elaborate iterative restoration methods. Also,

they are strictly linear by design. The reason behind that computational

efficiency lies in the fact that the computations are made in the Fourier

transform domain, where the 2-D matrix operations needed for inverting the

imaging equation are expressible as 1-D vector-only operations. Also, the

existence of very efficient FFT algorithms adds to the overall efficiency

of the technique.

The Wiener filter approach tries to build an "optimal" estimate of the

original, undegraded image, by enforcing a minimum mean-square-error

constraint between estimate and original images, under the assumption of

adequate spatial sampling and signal-independent noise [1]. A less strict

least-squares deviation criterion leads to the usual inverse filter. Under

the minimum mean-square-error constraint, the filter assumes a functional

form that involves essentially the blur matrix (PSF) and the original

image and noise autocorrelation matrices. Here we come to the first

problem: besides knowledge about the PSF, some a priori knowledge about

the original, undegraded image itself must be available, in order to

perform the restoration. This knowledge is unecessary in the case of

inverse filter, since it depends only on the PSF matrix. However, its

ill-conditioned behavior limits its use to very high signal-to-noise ratio

images.

For the most general case, a matrix inversion with size of the order of

N**2, where N is the number of pixels in the image, is required. This is

clearly an unfeasible task for reasonabily sized images, unless

simplifications can be found. Additional assumptions and approximations

about the structure of the original, undegraded image, as well as the PSF

nature, must be done.

The first approximation is to assume a spatially-invariant PSF. This

introduces so-called block-Toeplitz structure in the PSF matrix. A second

approximation is to assume that inter-pixel correlation is negligible

beyond a certain distance between pixels. Most important, however, is the

additional assumption that the image structure (texture) can be modelled

by a wide-sense stationary stochastic process; that is, a process whose

autocorrelation function is invariant with respect to spatial coordinate

translations. Also, constant mean must be assumed for this process. Under

all these assumptions, the autocorrelation matrices will also show

block-Toeplitz structure similar to the PSF matrix. For large sizes,

Toeplitz and block-Toeplitz matrices can be approximated by circulant and

block-circulant matrices, respectively. Since circulant matrices are

readily diagonalized by the Fourier transform matrix (that is, the Fourier

transform coefficients are the eingenvalues of a circulant [2]), the

problem reduces to a scalar inversion in the Fourier transform domain,

subject to a constraint dictated by the image and noise second-order

statistics. Notice that the circulant approximation is equivalent to

assuming periodicity in the image and PSF matrices.

Now lets resume the approximations adopted in deriving the

fast-computation Wiener restoration:

.ls (1)

Image and noise are wide-sense stationary with constant mean. Their

autocorrelation functions are known a priori.

.le

.ls (2)

PSF is spatially invariant.

.le

.ls (3)

Image and PSF are periodic.

.le

.ls (4)

PSF is not undersampled.

.le

.ls (5)

Noise is signal-independent.

.le

In the case of HST images, none of these assumptions are completely

satisfied. In particular, a typical astronomical image shows regions with

very different correlation functions: nearby a star image, inter-pixel

correlation is very high; in sky background regions correlation is low.

In other words, the stationarity hypothesis does not apply to those

images. This is the main reason why Wiener filter restorations look of

lower quality than more expensive methods, which in some way take into

account the spatially variant image statistics. Of course, a spatially

variant PSF and significant undersampling (as e.g. in the WF/PC)

also contribute to the

lower restoration quality atainable with Wiener restoration.

Another Wiener filter drawback comes directly from the minimum

mean-square-error constraint. It is known from psychophysics that the

human eye is not a minimum mean-square-error detector. The eye is willing

to accept more noise than allowed for by the Wiener filter, provided this

noise is spatially associated with sharp intensity changes. The

stationarity hypothesis plus the minimum mean-square-error constraint

result in restored images having an unpleasantly smooth look.

Nevertheless, experience with a limited number of HST images (mostly

pre-COSTAR FOC

f/96, where the PSF may be assumed to be spatially invariant, and is

adequately sampled) has shown that acceptable restorations can be atained

by the Wiener filter. For WF/PC images, lower quality results must be

expected, of course. The main advantage over iterative methods is

execution speed. In a Sparc 2, a 512 X 512 image may take 10-15 CPU

minutes to be deconvolved by the Richardson-Lucy algorithm, while the same

image is restored in 50-90 CPU seconds by the Wiener filter. One

may think of the Wiener filter

as a "quick-look" tool, capable of producing acceptable (not the best

possible) restorations in a very short time, while the final restoration

is performed by a costlier iterative algorithm.

.sh

2. FILTER IMPLEMENTATION

The most general filter implementation may be written as in [1]:

.nf

G(u,v) = F(u,v) * (I(u,v) ** alpha) * (W(u,v) ** (1.-alpha))

.fi

where G(u,v) is the estimated image Fourier transform,

F(u,v) is the input, degraded image Fourier transform,

I(u,v) is the inverse filter function,

W(u,v) is the Wiener filter function, and

alpha is a parameter.

For alpha=0 we have the standard Wiener filter, and for alpha=1.

the standard inverse filter. By varying alpha between 0. and 1.

we may emphasize the relative effect of each filter. The so-called

geometric mean filter is obtained setting alpha=0.5.

This geometric mean filter was introduced [1] as an attempt to

de-emphasize the low-frequency dominance of the Wiener filter,

while avoiding the early singularity of the inverse filter.

The inverse filter is simply:

.nf

H*(u,v)

I(u,v) = ----------------

(abs(H(u,v)))**2

.fi

and the Wiener filter is:

.nf

H*(u,v)

W(u,v) =  ---------------------------------------

(abs(H(u,v)))**2  + (Pn(u,v) / Pg(u,v))

.fi

where

H(u,v) is the PSF Fourier transform,

H*(u,v) its complex conjugate,

Pn(u,v) is the noise power spectrum, and

Pg(u,v) the original, undegraded image, power spectrum

(u,v is spatial frequency).

.sh

3. SIGNAL AND NOISE MODELS

The a priori knowledge about the undegraded image manifests itself in the

Pg(u,v) and Pn(u,v) terms, which are essentially the Fourier transforms of

the image and noise autocorrelation functions, respectively. The STSDAS

implementation provides a range of choices for these image and noise models.

.sh

3.1 Noise

Noise is assumed to be white (that is Pn(u,v) = Pn = constant), and is

measured at a certain fixed frequency

.nf

fnoise = sqrt (u**2 + v**2)

.fi

in the image Fourier transform F(u,v), averaging the power in all (u,v)

pixels satisfying relation above.  If not supplied by the user, the

program uses the largest value of fnoise available. This assumes that, at

these large frequencies, signal power is negligible with respect to noise

power. Notice that this may not be true in the case of undersampled

images.

It is possible to introduce a modification in the Wiener filter

derivation, to take into account signal-dependent noise [3,4]. The STSDAS

implementation includes that modification, for the case of Poisson noise.

This usually produces, when applicable, slight, but noticeabily better

restorations, than the standard independent noise model.

.sh

3.2 Signal

For the undegraded image model Pg(u,v), the available options are:

.ls white

This is the simplest image model. Pg(u,v) is assumed to be constant

and equal to the average power in the input image.

.le

.ls input

A good bet is to assume that the input image power spectrum is a

reasonable estimate of the undegraded image power spectrum. The observed

image power spectrum may be written in terms of the original image, PSF

and noise, as:

.nf

Pf(u,v) =~ (abs(H(u,v)))**2 * Pg(u,v) + Pn(u,v)

.fi

After subtracting the noise contribution, Pg(u,v) may be estimated from

the observed Pf(u,v). Using directly H(u,v)**2 usually induces strong high

frequency noise in the solution. A better approach is to simply multiply

Pf(u,v) by a "correlation parameter" > 1, to correct for the effects of

H(u,v)**2.  This approach usually works well in terrestrial scenes with

small blurs, where image correlation is large. In a typical HST stellar

field scene, however, where the input image power spectrum is dominated by

the PSF power spetrum, noticeable artifacts may be introduced in the

restoration.

.le

.ls Markov

It is known that, for terrestrial scenes, a first-order Markov stochastic

process is a good model for the image structure (texture) [5]. In such a

process, the autocorrelation matrix depends only on nearest-neighbor pixel

correlation. The program measures average inter-pixel correlation in the

input image and builds Pg(u,v) as a (circularly symmetric) Markov

spectrum. This usually produces the best restorations in terrestrial

scenes. In HST images there is a tendency to strongly low-pass filter the

image. This is because the average inter-pixel correlation in a scene

dominated by sky background is very small. Under such conditions, the

computed Markov spectrum will show very low powers at high spatial

frequencies, thus strongly de-emphasizing these frequencies. The

geometric filter form is a useful compromise in compensating that behavior.

.le

.ls Gaussian

In some circunstances, we may be interested in restoring optimally some

specific structure in the image. In the particular case of HST stellar

fields, this particular structure may be a star image. We may adopt as

signal model the shape of such a star image; in the current

implementation, the adopted model is a Gaussian shape. Notice that this

will produce non-optimal restoration of the sky background, effectively

enhancing the high frequency noise in that background. Nevertheless, it

may be the best signal model when we are interested in restoring star

images in a crowded field with a minimum amount of contamination [6].  It

may be argued that the best model would be instead an Airy diffraction

pattern compatible with the telescope pupil characteristics. Experiments

with this image model showed an intolerable amount of "ringing" around

star images. This is because the noise suppression mechanism in the Wiener

filter introduces such ringing associated with abrupt intensity changes

[7]. This mechanism enhances the already present ring structure in the

diffraction pattern.

.le

.ls psf

As a compromise between the Gaussian signal model, and the input image

signal model (for HST images), we may take the degraded star, that is, the

PSF itself, as signal model.

.le

.ls external image

In some circunstances, there may be available other images which belong to

the same "ensemble" as the image being restored. More specifically, an

external image with similar second-order statistics as the one being

restored is available. This image may be used as source for the signal

model Pg(u,v).

.le

There is one more implemented functional form for the Wiener filter, the

so called parametric filter. It completely ignores the points discussed

above, and uses as substitute for the term Pn(u,v)/Pg(u,v) a parameter

gamma. Small values of gamma will enhance the high frequency content on

the restored image; large values will enhance the low-pass filter

characteristics.  In the current implementation, the value of gamma input

by the user is actually multiplied by the average of Pn(u,v)/Pg(u,v), with

both power spectra measured in the input image. Gamma thus represents the

input image average noise-to-signal ratio in frequency space.

.sh

4. HINTS ON USAGE

First, the user must be aware that the restoration quality depends,

more than everything else, on the quality of the available PSF image.

This is related to the problem of so-called consistent convolutions [8].

Regarding signal model choice, the best approach when trying to deconvolve

an image for the first time is to leave all filter settings at their

default values. These include: ftype=Wiener, signalm=white (notice that

this is precisely the same filter generated by the combination

ftype=parametric, gamma=1.). Another suitable starting point could be

ftype=geometric, signalm=Markov. Both of these settings have the advantage

of being independent of any numerical input parameter, and are

representative of the "average" filter for the image being studied.

If the goal is to restore stellar images, the Gaussian signal model should

be tried. To minimize ringing around very bright stars, as well as sky

background noise, a mild (fwhm ~ 1.5 - 2 pixels) low-pass filter may be

useful. Another way to avoid ringing it is to resort to the inverse plus

low-pass filter [7]. However, stellar shapes will become more erratic

along the image.

For extended object restoration, a good bet is the ftype=geometric,

signalm=Markov model.

As a last comment, we must stress that most of the results obtained with

the more sophisticated signal models could be approximatelly arrived at by

the simpler parametric and pure inverse filters, if coupled to a suitably

chosen low-pass filter. The values of gamma and fwhm have to be chosen by

trial and error, until a "satisfactory" image is obtained.

This approach has two disadvantages relative to the more elaborate

filters: the results are only approximately equal; and, we do not know

precisely which criteria is being used for the restoration. In other

words, we do not know how far, and for which spatial structures and

frequencies, we are from the optimal minimum mean-square-error criterion.

.sh

5. REFERENCES

.ls [1]

ANDREWS, H.C., HUNT, B.R., 1977, "Digital Image Restoration",

Prentice-Hall, New Jersey.

.le

.ls [2]

HUNT, B.R., 1971, "A Matrix Theory Proof of the Discrete Convolution

Theorem", IEEE Trans. Audio Electroacustics, v.AU-19, p.285

.le

.ls [3]

KONDO, K., ICHIOKA, Y., SUZUKI, T., 1977, "Image Restoration by Wiener

Filtering in the Presence of Signal-dependent Noise", Applied Optics,

v.16, p.2554

.le

.ls [4]

WHAL, F.M., 1987, "Digital Image Signal Processing", Artech House, London.

.le

.ls [5]

PRATT, W.K., 1978, "Digital Image Processing", John Wiley, New York.

.le

.ls [6]

BUSKO, I.C., 1991, "Wiener Restoration of HST Images: Signal Models

and Photometric Behavior", in: First Annual Conference on Astronomical Data

Analysis Software and Systems, Tucson.

.le

.ls [7]

LAGENDIJK, R.L, BIEMOND, J., BOEKEE, D.E., 1986, "Iterative Image

Restoration with Ringing Reduction", in: Signal Processing III: Theories

and Applications, I.T. Young et.al. (editors), Elsevier Science Publishers

B.V., North-Holland.

.le

.ls [8]

BATES, R.H.T., McDONNELL, M.J., 1986, "Image Restoration and Reconstruction",

Claredon Press, Oxford.

.le

.sh

6. AUTHOR

.nf

Ivo Busko

Astrophysics Division

National Space Research Institute, Brazil

busko@das.inpe.br

.fi

.endhelp